CONCRETE PRE-POUR CHECKLIST

| Ref. No.: |  | Proposed Pour Date: |  |
| --- | --- | --- | --- |
| Contract Specifications Clause: |  | Type of Structure: |  |
| Location: |  | Type/Strength of Concrete: |  |
| Drawing Ref.: |  | Concrete Quantity: |  |

|  | Yes | No | N/A | NCR |
| --- | --- | --- | --- | --- |
| 1.0 Excavation |  |  |  |  |
| 1.1 Is the excavation correct in size? |  |  |  |  |
| 1.2 Is the base correctly compacted? |  |  |  |  |
| 1.3 Is the excavation dry? |  |  |  |  |
| 1.4 Is selected fill/blinding concrete in place? |  |  |  |  |
| 2.0 Formwork |  |  |  |  |
| 2.1 Is bracing adequate? |  |  |  |  |
| 2.2 Has mold oil been applied evenly and thoroughly? |  |  |  |  |
| 2.3 Is formwork vertical? |  |  |  |  |
| 2.4 Is horizontal alignment correct? |  |  |  |  |
| 2.5 Is access adequate? |  |  |  |  |
| 3.0 Reinforcement |  |  |  |  |
| 3.1 Are the rebars free from excessive rust? |  |  |  |  |
| 3.2 Is the spacing correct? |  |  |  |  |
| 3.3 Is the concrete cover as per specs? |  |  |  |  |
| 3.4 Are all tie wires in place and correctly tied? |  |  |  |  |
| 3.5 Are there adequate spacers/chairs at specified intervals? |  |  |  |  |
| 3.6 Correct number of bars in the reinforcement. |  |  |  |  |
| 3.7 Are all the bars of the correct size? |  |  |  |  |
| 3.8 Are the starter bars in the correct position? |  |  |  |  |
| 3.9 Are the starter bars and splices the correct size and number? |  |  |  |  |
| 4.0 General |  |  |  |  |
| 4.1 Is the pour site clean and free from all debris? |  |  |  |  |
| 4.2 Are there adequate vibratory pokers on site? |  |  |  |  |
| 4.3 Is curing material available on site? |  |  |  |  |
| 4.4 Have slump tests and concrete sampling been organized? |  |  |  |  |
| 4.5 Has adequate lighting been organized? |  |  |  |  |
| 4.6 Are stop blocks on site? |  |  |  |  |
| 4.7 Is the access safe? |  |  |  |  |
| 4.8 Other/Special Requirements. |  |  |  |  |

| Contractor: | Employer: |
| --- | --- |
| __________________________ | __________________________ |